# Engineering Investigation & Research

## 2.1 Preparation

## 2.1.1 Purpose

The purpose of the Engineering Investigation & Research stage is to establish the technical understanding required to design and validate the fictional Operational Technology work package.

The investigation will answer the engineering questions necessary to define:

- the simplified distribution network;

- the expected behaviour following the feeder fault;

- the roles of the represented OT systems;

- the information exchanged between those systems;

- the logic used to identify the outage and affected customers;

- the conditions under which restoration may be proposed or rejected;

- the treatment of telemetry quality and uncertainty;

- the operational event record;

- and the basis for the intentional validation defect.

The investigation is not intended to establish expert-level knowledge of electricity distribution engineering or commercial SCADA/ADMS implementations. Research depth should be proportional to the decisions required by this work package.

That boundary follows the governing project’s explicit intention to demonstrate understanding of system interaction rather than reproduce commercial software.

## 2.1.2 Investigation Questions

I think we should formally establish the investigation questions before researching them.

### IQ-01 — What network information must be represented?

We need to determine the minimum information required to model the fictional network defined by the project:

one zone substation → two primary feeders → distribution sections → normally-open tie switch → customer zones → simplified loads.

Questions include:

What properties must a feeder, breaker, switch, network section and customer zone possess?

How should connectivity be represented?

How do switch/breaker states determine whether a section is energised?

This investigation will directly inform the Network Model.

### IQ-02 — What should happen when the feeder fault occurs?

The Project Definition already gives us the authoritative high-level sequence:

Fault → protection trip → topology update → de-energisation → alarm → outage identification → affected-customer calculation → operator acknowledgement → restoration proposal → capacity check → restoration permitted/rejected → event logging.

We therefore don’t need research to invent another workflow.

We need research to understand what each stage actually means well enough to model it credibly.

For example:

- What changes when the feeder breaker opens?

- How is downstream energisation inferred?

- What constitutes an alarm versus an outage?

- How might an operator become involved?

- What information is required before restoration can be considered?

This investigation informs the Operational Workflow and later the Requirements Specification.

### IQ-03 — What role does SCADA play?

We need enough understanding of SCADA to determine what information our conceptual SCADA representation provides to the workflow.

Likely areas requiring investigation include:

- breaker and switch status;

- telemetry;

- alarms/events;

- remote indications;

- data quality/state;

- operator interaction.

Importantly, we don’t need to investigate how to build SCADA.

The engineering question is:

What information would the other operational functions need from SCADA during this scenario?

### IQ-04 — What role does ADMS play?

Because the fictional project represents an upgraded Distribution Control Environment, we need to understand which parts of our workflow reasonably belong to an ADMS-type capability.

Relevant questions include:

- How is network topology represented?

- How is energisation state derived?

- How might operational applications use network connectivity?

- How might restoration recommendations be generated?

- What information must be available before a switching/restoration recommendation is considered valid?

This will become particularly important when defining the boundary between observed telemetry and derived network state.

### IQ-05 — What role does OMS play?

The scenario requires an outage to be identified and affected customers calculated.

We therefore need to understand at a conceptual level:

- how an outage relates to the electrical network model;

- how de-energised sections map to affected customers;

- what information OMS might receive from other operational systems;

- and what outage information might be returned to operators or other systems.

Again, we’re modelling the role, not reproducing a commercial OMS.

### IQ-06 — What role does GIS play?

GIS is explicitly included among the operational concepts represented by the project.

The key engineering question is likely:

What network and asset information originates from, or is maintained in relation to, GIS, and how does that information support the operational network model?

This matters because our intentional defect may eventually involve configuration/topology information.

### IQ-07 — How should network topology and energisation be determined?

This is one of the most important investigations.

We need a defensible simplified rule for determining whether each network section is energised based on:

- connectivity;

- source;

- breaker state;

- switch state;

- and possibly telemetry validity.

For example, conceptually:

A section is energised when a valid electrical path exists from an energised source through connected closed devices.

But we shouldn’t lock the exact model until we’ve investigated the concept sufficiently.

This will become the engineering basis for topology validation.

### IQ-08 — How should affected customers be calculated?

The governing documents require the system to identify affected customers following the fault.

We need to decide whether our simplified model represents:

- individual customers;

- customer counts attached to network sections;

- customer zones;

- or another appropriately simplified representation.

Given the project scope, we probably don’t need individual customer records, but that is a design conclusion we should reach through this investigation, rather than casually assume.

### IQ-09 — What constitutes a valid restoration?

The Project Definition gives us an important constraint: restoration must only occur where electrically valid, supported by simplified analysis of feeder loading, transferable load, alternate-feeder capacity and resulting loading percentage.

We need to establish:

- what load is being transferred;

- what alternate feeder supplies it;

- what switching arrangement enables that transfer;

- what capacity limit applies;

- how resulting loading is calculated;

- and what conditions cause restoration to be rejected.

This will eventually produce one of the project’s most visible pieces of actual engineering reasoning.

### IQ-10 — How should stale or uncertain telemetry affect operations?

This comes directly from the Project Vision, which says that uncertain or stale telemetry should prevent unsafe actions.

We need to establish a simplified but defensible treatment of telemetry quality.

For example:

- What constitutes “stale”?

- Which telemetry matters to the restoration decision?

- Should stale telemetry prevent topology determination, restoration recommendation, switching action, or some combination?

- How should the condition be communicated to the operator?

This deserves explicit investigation because it demonstrates something very important about OT engineering:

having a value is not necessarily the same thing as being able to trust that value.

### IQ-11 — What events need to be logged?

The project requires operational actions to be traceable through system logs.

We need to establish a sensible minimum event record.

Potential information includes:

- timestamp;

- event source;

- event type;

- affected asset;

- previous state;

- new state;

- operator/system origin;

- description;

- validation relevance.

This investigation will later influence both the system design and validation evidence.

### IQ-12 — What should the intentional engineering defect be?

The governing documents deliberately leave several possibilities open:

- incorrect switch normal state;

- incorrect topology connection;

- stale telemetry;

- inconsistent configuration data.

This is therefore one of the few important project decisions that is genuinely still open.

We should not choose it based on which bug is easiest to code.

Instead, we should investigate which defect produces the strongest and most realistic graduate engineering exercise:

failed requirement → observable symptom → investigation → evidence → root cause → configuration correction → repeat test → successful verification.

I recommend that we do not lock the defect yet. We should choose it after investigating topology, telemetry and system information flows because then we’ll understand which defect produces the most believable engineering investigation.

## 2.1.3 Investigation Outputs

Rather than producing dozens of pages of general research, each investigation topic should result in a compact engineering record containing:

Question → Relevant Findings → Project Interpretation → Engineering Decision/Assumption → Affected Deliverables

For example, later we might have something like:

Investigation: Network energisation determination
Finding: Operational network state depends on connectivity and switching-device state.
Project interpretation: Full electrical power-flow modelling is unnecessary for the defined validation scenario.
Decision: Energisation will be determined using connectivity from an available source through closed devices.
Affected artefacts: Network Model, Requirements Specification, Operational Workflow, validation tests.

That is much more valuable for this project than a five-page essay explaining what SCADA is.

It creates an explicit chain of:

research → understanding → engineering decision → implementation → validation.

And that’s exactly the type of traceability we’re trying to demonstrate.

## How I suggest we execute Step 2

We shouldn’t attempt all twelve investigations simultaneously.

There are dependencies between them. I recommend working through them in four small investigation packages:

Package A — Distribution Network & Topology: IQ-01 network information, IQ-07 topology/energisation, IQ-08 affected customers.

Package B — Operational Systems & Information Flow: IQ-03 SCADA, IQ-04 ADMS, IQ-05 OMS, IQ-06 GIS, IQ-11 event logging.

Package C — Fault & Restoration Workflow: IQ-02 fault behaviour, IQ-09 restoration validity, IQ-10 telemetry quality.

Package D — Defect Selection: IQ-12, using everything learned in A–C to choose and formally justify the intentional engineering defect.

That sequence means we’re not researching randomly. Each package answers questions required by the next stage, and at the end of Step 2 we’ll have enough evidence to begin Step 3 — Engineering Design Brief

## 2.2 Engineering Investigation A

Good. For this stage I did a targeted external investigation rather than relying only on general knowledge. The governing documents tell us what the fictional network must achieve; external sources are being used only to establish a technically defensible simplified model.

# Engineering Investigation A

## Distribution Network, Connectivity & Topology

Investigation ID: EI-A
Status: Draft
Related Investigation Questions: IQ-01, IQ-07, IQ-08

# A.1 Purpose

The purpose of this investigation is to determine the minimum distribution-network information and topology behaviour required to support the project’s feeder-fault validation scenario.

The governing Project Definition establishes a deliberately simplified network containing:

- one zone substation;

- two primary feeders;

- several distribution sections;

- a normally-open tie switch;

- customer zones;

- simplified electrical loads; and

- simplified telemetry points.

The network must support validation of downstream de-energisation, topology updates, affected-customer calculation and restoration subject to simplified electrical constraints.

The investigation therefore addresses three questions:

IQ-01: What network information must be represented?

IQ-07: How should network topology and energisation be determined?

IQ-08: How should affected customers be calculated?

# A.2 Investigation Boundary

This investigation does not attempt to design a realistic utility distribution network in full detail.

Real distribution networks may contain transformers, laterals, fuses, reclosers, voltage regulators, capacitor banks, multiple voltage levels, phases, distributed generation and numerous other assets. U.S. Department of Energy material, for example, describes feeder primaries with branches and laterals, distribution transformers and secondary circuits, while also noting that feeder circuits are frequently reconfigured for maintenance, load balancing, fault isolation and service restoration.

Most of that detail does not contribute to our defined validation problem.

The investigation therefore seeks the minimum credible network abstraction needed to reason about:

source → connectivity → switching state → energisation → outage extent → customers affected → restoration path.

That is the engineering problem our model must support.

# A.3 IQ-01 — Required Network Information

## Finding A1 — A radial operating structure is appropriate

Distribution feeders are commonly operated radially: a particular part of the network normally receives supply through one electrical path from its source. EPRI describes primary distribution systems as predominantly radial and notes that adjacent feeders can be connected through a normally-open tie switch while retaining radial operation under normal conditions.

DOE material similarly describes distribution feeder circuits as frequently being reconfigured for fault isolation and service restoration.

This strongly supports the topology already implied by our governing documents.

### Project interpretation

Our fictional network should operate in a simple radial configuration under normal conditions, with:

- Feeder A supplying its own sections;

- Feeder B supplying its own sections;

- the tie switch between them normally open.

We do not need a meshed network.

This gives us a simple but legitimate engineering basis for understanding what happens when a feeder loses supply.

### Engineering decision — ED-A01

The fictional distribution network shall normally operate radially, with each network section having one active supply path under normal operating conditions.

# A.4 Required Asset Types

We now need to decide what objects must actually exist in the model.

I recommend six principal engineering object types.

### 1. Source / Zone Substation

We need a representation of the upstream source from which the two feeders are supplied.

Because substation design itself is outside scope, we do not need to model busbars, incoming transmission circuits or transformer internals unless later investigation demonstrates that one of these is required.

For our purposes, the zone substation represents the available upstream supply.

### 2. Feeder

Each primary feeder represents a distribution circuit originating from the zone substation.

At minimum, each feeder needs:

- unique identifier;

- name;

- source;

- operating capacity;

- present loading;

- associated network sections.

Capacity and loading are required because the governing Project Definition explicitly requires restoration reasoning based on feeder loading, transferable load, alternate-feeder capacity and resulting loading percentage.

### 3. Feeder Breaker

Each feeder requires a breaker at its source.

For our purposes, the essential property is:

OPEN / CLOSED

We do not need to model relay settings, interrupting ratings or protection curves.

The breaker matters because the governing workflow explicitly establishes:

fault occurs → protection trips feeder breaker → network topology updates.

The breaker therefore forms part of the operational topology rather than merely being a visual asset.

### 4. Distribution Section

This should become one of the most important objects in our model.

A section represents a meaningful portion of the feeder whose connectivity and energisation state can be determined.

At minimum, a section needs:

- unique identifier;

- name;

- feeder association;

- connectivity to adjacent network elements;

- simplified load;

- energisation state;

- associated customer zone.

This gives us a manageable abstraction without modelling every conductor, pole and transformer.

### 5. Tie Switch

The normally-open tie switch connects appropriate sections of Feeder A and Feeder B.

It requires at minimum:

- unique identifier;

- connected network elements;

- normal state;

- current state.

Under normal operation:

Tie = OPEN

During an appropriate restoration scenario:

Tie = CLOSED

The physical principle is credible: PNNL describes normally-open connections between adjacent feeders being closed after a fault has been located and isolated, allowing intact portions of a damaged feeder to be supplied temporarily through another path.

This is almost exactly the behaviour our fictional scenario needs.

### 6. Customer Zone

Rather than representing individual houses or accounts, we should associate groups of customers with network sections.

At minimum:

- unique identifier;

- zone name;

- connected section;

- customer count.

Potentially also:

- aggregated load.

I’ll justify this decision properly under IQ-08.

# A.5 Preliminary Network Object Model

We can therefore already establish a conceptual relationship:

ZONE SUBSTATION

│

├──────── FEEDER A BREAKER

│               │

│          FEEDER A

│               │

│        Distribution Sections

│               │

│          Customer Zones

│

└──────── FEEDER B BREAKER

│

FEEDER B

│

Distribution Sections

│

Customer Zones

Selected section ── TIE SWITCH ── Selected section

Feeder A                 Feeder B

This is not yet our final network diagram.

We haven’t decided how many sections each feeder contains, where the fault occurs or exactly where the tie connects. Those are Step 5 Network Model decisions.

At this stage we’re establishing the information model required to make those later decisions.

### Engineering decision — ED-A02

The network model shall represent, at minimum, an upstream source, two primary feeders, feeder breakers, distribution sections, one normally-open tie switch and aggregated customer zones.

This is sufficient to represent the scenario required by the governing documents without introducing unnecessary physical-network complexity.

# A.6 IQ-07 — Topology and Energisation

This is the most important conclusion of Investigation A.

We need to distinguish two concepts:

### Connectivity

Can an electrical path exist between these network elements given their configured relationships?

and:

### Energisation

Given the current switching states, does an active path currently exist from an available source to this section?

That distinction will become extremely useful later.

A section can remain physically connected in the model while being de-energised operationally because an upstream breaker has opened.

# A.7 Simplified Energisation Rule

For this project, I recommend the following engineering rule:

A distribution section is considered energised when a continuous path exists from an available source to that section through the configured network topology, with every switching device on the active path in a state that permits supply.

This gives us a clean topology-based method.

Conceptually:

Source

↓

Breaker CLOSED?

↓

Section 1

↓

Section 2

↓

Section 3

If the breaker opens:

Source

↓

Breaker OPEN

X

Section 1    DE-ENERGISED

↓

Section 2    DE-ENERGISED

↓

Section 3    DE-ENERGISED

We do not need to solve electrical voltages or power flows to establish this basic operational state.

That is an appropriate simplification because our project is validating OT topology behaviour rather than performing a detailed network study.

### Engineering decision — ED-A03

Section energisation shall be determined using topology tracing from an available source through connected network elements and switching devices whose current states permit supply.

# A.8 Topology Must Be Dynamic

This produces another important conclusion.

We should not store “Feeder A Section 3 is energised” as an unquestioned permanent property.

Its energisation state depends on the network configuration.

Consider:

NORMAL

SOURCE

│

[A BREAKER — CLOSED]

│

A1

│

A2

│

A3

│

[TIE — OPEN]

│

B3

│

B2

│

[B BREAKER — CLOSED]

│

SOURCE

A1–A3 are supplied from Feeder A.

B1–B3 are supplied from Feeder B.

Now suppose Feeder A loses its normal supply.

[A BREAKER — OPEN]

A1

│

A2

│

A3

NO ACTIVE SOURCE PATH

Those sections become de-energised.

If an appropriate isolated portion is subsequently connected through the tie:

SOURCE B

│

B FEEDER

│

B3

│

[TIE — CLOSED]

│

A3

│

A2

then those intact sections may once again have an active source path, now from Feeder B.

This behaviour is consistent with PNNL’s description of adjacent-feeder ties being used to temporarily power intact portions of a faulted feeder.

### Engineering decision — ED-A04

Energisation shall be treated as a derived operational state rather than a fixed characteristic of a network section.

That’s a small design decision with a lot of value. It means the demonstrator behaves according to the network rather than according to hard-coded screen states.

# A.9 Important Restoration Implication — Fault Isolation

There is one subtle issue we need to capture now.

We cannot simply do:

Feeder A breaker trips → close tie → power everything from Feeder B.

If the fault remains electrically connected, restoring supply through another feeder could simply energise the fault again.

Industry material describes the principle as locating and isolating the fault before intact portions are supplied from an adjacent feeder through a normally-open connection.

This means our eventual network needs some mechanism for separating the faulted section from the healthy section we want to restore.

That could eventually mean a sectionalising switch or another switchable network boundary.

However, I do not recommend locking the exact device arrangement during Investigation A. That belongs in the Network Model/design work.

The important finding is:

### Engineering requirement implication — RI-A01

Restoration must not establish an energised path through the identified faulted section. The faulted portion of the network must remain isolated before alternate supply is established.

This will later become a formal requirement.

This is a good example of why we did engineering investigation before drawing the network. Otherwise we might have created a visually plausible tie switch arrangement that was operationally nonsensical.

# A.10 IQ-08 — Affected Customers

The governing documents require the validation to confirm that affected customers are calculated correctly.

In real systems, the relationship between customers and the electrical network can be much more detailed. Esri material describes outage-management network tracing being used to determine outage extents and affected customers, and another utility case study notes that correct customer-to-network linkage is important for outage management.

We do not need that level of customer modelling.

Our engineering question is simply:

Which customer groups are supplied by the network sections that have become de-energised?

# A.11 Customer Aggregation Decision

I recommend:

Customer counts will be associated with defined customer zones, with each customer zone connected to a distribution section.

For example:

Section A1

└── Customer Zone A1

└── 420 customers

Section A2

└── Customer Zone A2

└── 310 customers

Section A3

└── Customer Zone A3

└── 185 customers

If A2 and A3 become de-energised:

Affected customers

= 310 + 185

= 495

We don’t need 495 individual database records to demonstrate that reasoning.

Esri describes GIS/network information being used to determine connected or downstream customers during outages, including utilities using downstream tracing to identify affected customers.

Our aggregation preserves the engineering relationship while avoiding irrelevant customer-management complexity.

### Engineering decision — ED-A05

Customers shall be represented as aggregated customer zones associated with distribution sections rather than as individual customer records.

# A.12 Affected-Customer Rule

This gives us another explicit rule:

A customer zone is considered affected by an outage when the distribution section supplying that zone is de-energised.

Therefore:

N_{affected}=\sum_{i=1}^{n}N_i

where N_i is the customer count associated with each de-energised customer zone.

If restoration re-energises a section, customers attached to that section are no longer counted as currently affected.

This gives us an elegant connection:

switch state → topology → section energisation → customer impact

rather than:

fault button → display arbitrary customer number.

That difference is important to the engineering credibility of the demonstrator.

### Engineering decision — ED-A06

Affected-customer count shall be derived from the customer zones associated with currently de-energised network sections.

# A.13 Investigation A — Resulting Engineering Model

We can now describe the minimum behaviour our future network model must support:

NETWORK CONFIGURATION

+

CURRENT SWITCH/BREAKER STATES

↓

TOPOLOGY TRACE

↓

ACTIVE SOURCE PATHS

↓

SECTION ENERGISATION STATES

↓

DE-ENERGISED SECTIONS

↓

ASSOCIATED CUSTOMER ZONES

↓

AFFECTED CUSTOMER COUNT

And during restoration:

FAULT ISOLATED

+

TIE SWITCH CLOSED

+

ALTERNATE SOURCE AVAILABLE

+

RESTORATION CONSTRAINTS SATISFIED

↓

NEW ACTIVE SUPPLY PATH

↓

TOPOLOGY RECALCULATED

↓

HEALTHY SECTIONS RE-ENERGISED

↓

CUSTOMER OUTAGE COUNT UPDATED

That is essentially the engineering core of our eventual demonstrator.

# A.14 Investigation Conclusions

Investigation A results in six engineering decisions and one requirement implication:

| ID | Conclusion |
| --- | --- |
| ED-A01 | Network normally operates radially. |
| ED-A02 | Model source, two feeders, feeder breakers, distribution sections, normally-open tie and customer zones. |
| ED-A03 | Determine energisation through topology tracing from an available source through permissible device states. |
| ED-A04 | Energisation is derived from current topology rather than permanently assigned to sections. |
| ED-A05 | Represent customers as aggregated customer zones associated with sections. |
| ED-A06 | Calculate affected customers from customer zones attached to de-energised sections. |
| RI-A01 | A faulted section must remain isolated before alternate supply can restore healthy sections. |

These decisions are consistent with the governing project’s intentionally small network while giving us enough engineering behaviour to validate topology updates, outages, customer impacts and restoration.

## Deliverable impact

The conclusions from Investigation A will later feed directly into:

Engineering Design Brief: radial topology, network abstraction and restoration philosophy.

Requirements Specification: topology, energisation, customer-impact and restoration-isolation requirements.

Network Model: actual feeder/section/switch arrangement and numerical values.

Operational Workflow: topology recalculation following switching operations.

Validation Plan: tests proving that breaker/switch states produce the expected energisation and customer-impact results.

Demonstrator: topology engine and network visualisation.

There is one especially useful outcome from this investigation: we’ve discovered something that wasn’t explicit enough in the original planning documents—the need for fault isolation before restoration. That’s exactly the kind of legitimate refinement Step 2 is supposed to uncover. It doesn’t overturn a locked project decision; it makes the existing restoration scenario technically coherent.

I would not design the actual TasGrid network yet. We now understand what the network must be capable of doing, but Package B should establish what SCADA, ADMS, OMS and GIS are responsible for and where this topology/customer information comes from. Once B and C are complete, we’ll be in a much stronger position to design the actual network without prematurely assigning responsibilities to the wrong conceptual systems.

Absolutely. Package B is where we define who does what across the conceptual OT environment. This matters because if we blur SCADA, ADMS, OMS and GIS together, the demonstrator may still “work,” but it will teach and demonstrate the wrong systems thinking.

# Engineering Investigation B

## Operational Systems & Information Flow

Investigation ID: EI-B
Status: Draft
Related Investigation Questions: IQ-03, IQ-04, IQ-05, IQ-06, IQ-11

# B.1 Purpose

The purpose of this investigation is to determine the conceptual roles of SCADA, ADMS, OMS and GIS within the project’s feeder-fault validation scenario, and to define the minimum information exchanges needed between them.

The governing Project Definition explicitly states that these systems are to be represented conceptually only, with the objective being to demonstrate understanding of how they interact rather than replicate commercial software.

OT project definition.pdf

The investigation therefore focuses on one question:

What information does each system contribute to the end-to-end operational workflow, and what should remain outside its responsibility?

# B.2 System Boundary Philosophy

Real utility implementations vary significantly.

An ADMS may incorporate functions that historically existed in separate SCADA, DMS or OMS products. Some utilities operate highly integrated platforms; others maintain separate systems connected through interfaces. NREL describes ADMS as an integrated platform combining utility systems and supporting automated outage restoration and distribution-grid optimisation.

Therefore, it would be misleading for this fictional project to claim there is one universally correct product architecture.

Instead, we will use functional boundaries.

That means:

- SCADA represents field-state acquisition and operational alarms;

- GIS represents the maintained network/asset connectivity model;

- ADMS represents the operational network model and topology-based analysis;

- OMS represents outage and customer-impact information;

- the demonstrator represents the interaction between those conceptual functions.

### Engineering decision — ED-B01

System responsibilities shall be defined functionally rather than by attempting to reproduce the architecture of a specific commercial OT platform.

This preserves technical credibility while remaining within scope.

# B.3 IQ-03 — SCADA Role

SCADA is the part of our conceptual environment closest to the field equipment.

For this project, SCADA should answer:

What is the current observed state of monitored equipment, and what operational conditions have been reported?

It should not determine the entire outage or restoration strategy.

# B.4 SCADA Information Relevant to the Scenario

At minimum, our conceptual SCADA representation should provide:

- feeder breaker status;

- relevant switch status;

- telemetry timestamps or quality;

- protection/field indications relevant to the feeder event;

- alarms generated from operational conditions;

- operator acknowledgement where applicable.

Commercial SCADA products also log point updates, alarms, user interaction and other system occurrences as events, creating an operational audit trail.

For our scenario:

FIELD / PROTECTION

↓

Breaker opens

↓

SCADA receives breaker status

↓

SCADA records state change

↓

Alarm/event becomes available

↓

Operational applications consume updated state

This is a much better representation than having ADMS spontaneously “know” that a breaker tripped.

### Engineering decision — ED-B02

SCADA shall represent the acquisition of monitored field states and operational events required by the validation scenario.

# B.5 SCADA Does Not Own Derived Topology

There is an important distinction here.

SCADA may tell us:

Feeder A breaker = OPEN.

But that is not the same statement as:

Sections A1, A2 and A3 are de-energised and 915 customers are affected.

The first is an observed equipment state.

The second is a derived operational consequence requiring network connectivity information.

This distinction is central to the project.

### Engineering decision — ED-B03

SCADA shall provide observed device states; downstream network energisation shall be derived by the operational network model rather than treated as direct SCADA telemetry.

This will later give us a useful validation chain:

Breaker telemetry

↓

Network topology calculation

↓

Section energisation

↓

Outage extent

↓

Affected customers

# B.6 IQ-06 — GIS Role

GIS should answer:

What assets exist, how are they geographically/logically connected, and what persistent network configuration is maintained?

GIS is not just a background map in this project.

Esri describes utility-network GIS as supporting connectivity, tracing of electricity through network features and an operational view of how dynamic devices are configured.

Operational utility implementations also use GIS-maintained network models across outage management, distribution automation and distribution-management teams.

For our project, GIS can conceptually provide:

- asset identifiers;

- network sections;

- feeder associations;

- switch locations;

- configured network connectivity;

- customer-zone connectivity;

- persistent/normal equipment configuration.

# B.7 GIS Versus Live Operational State

We need another important distinction.

GIS might say:

Tie switch TS-01 connects Section A3 to Section B3.

That is configuration/connectivity information.

SCADA might say:

TS-01 current state = OPEN.

That is live operational information.

Combined, they tell us:

A physical/configured connection exists, but it is not currently an active electrical path.

This separation is extremely useful because it gives us a believable way to model configuration defects.

A wrong GIS/topology relationship could exist even though the SCADA switch state itself is perfectly correct.

Historical utility integration literature explicitly identifies network connectivity information from GIS as important to DMS functions.

### Engineering decision — ED-B04

GIS shall represent persistent network asset and connectivity information, while current switch/breaker states shall be supplied through the operational telemetry representation.

# B.8 IQ-04 — ADMS Role

For our project, ADMS should answer:

Given the configured network and the current operating states, what is the operational state of the distribution network, and what actions are permissible?

This is where Investigation A connects directly to Package B.

ADMS uses the network model together with operational state information to support situational awareness and distribution-system analysis. NREL describes ADMS as integrating multiple utility functions and supporting capabilities including fault location, isolation and service restoration.

Our conceptual ADMS does not need advanced optimisation.

It needs only the functions relevant to our scenario.

# B.9 Minimum ADMS Functions for This Project

I recommend representing five functions.

### 1. Operational topology processing

Combine:

configured connectivity + current device state

to determine:

active network topology.

### 2. Energisation determination

Determine which distribution sections currently have an active source path.

This is ED-A03 from Investigation A.

### 3. Faulted/isolation awareness

Track which part of the network is unavailable or must remain isolated during restoration.

This supports RI-A01 from Investigation A.

### 4. Restoration assessment

Evaluate whether healthy de-energised sections may be restored through the normally-open tie.

This will later incorporate:

- correct isolation;

- valid telemetry;

- alternate source availability;

- capacity constraint.

### 5. Operational topology update following switching

After an accepted switching state change:

switch state changes

↓

topology recalculated

↓

energisation recalculated

↓

outage/customer state recalculated

### Engineering decision — ED-B05

ADMS shall conceptually own operational topology processing, section energisation determination and simplified restoration assessment.

# B.10 ADMS Is the Integration Point — But Not the Owner of Everything

For our simplified project, it is reasonable to treat ADMS as the main operational integration point.

Conceptually:

GIS

(network configuration)

\

\

→ ADMS ← SCADA

/        (live state)

/

OMS

(customer/outage relationship)

But we should not collapse the systems into one database conceptually.

The project has much more learning value if we can say:

ADMS derives operational state using network information maintained by GIS and live state information obtained through SCADA, then contributes topology/outage information to the wider operational workflow.

That demonstrates system integration rather than just application logic.

# B.11 IQ-05 — OMS Role

OMS should answer:

Which customers are affected, and what is the current outage state?

Outage management systems integrate network and operational information to support outage identification and restoration workflows. Older but useful utility integration literature shows OMS-type functionality integrating information from GIS, SCADA and customer systems to support outage diagnosis and restoration.

For our simplified project, OMS does not need:

- call-centre management;

- crew dispatch;

- estimated restoration times;

- public notifications;

- customer communications;

- field work management.

Those would be realistic OMS functions in a broader project but are outside our defined scenario.

# B.12 Minimum OMS Functions

Our OMS representation should do only three things.

### 1. Receive/consume outage extent

It needs to know which network sections are currently de-energised.

### 2. Associate affected sections with customer zones

Using the customer-zone relationship established in Investigation A.

### 3. Produce outage/customer impact information

For example:

Outage O-001

Affected sections:

A2

A3

Affected customer zones:

CZ-A2

CZ-A3

Affected customers:

495

### Engineering decision — ED-B06

OMS shall represent outage identification and customer-impact calculation using de-energised sections and their associated customer zones.

# B.13 Where Should Customer Connectivity Live?

There is a small architectural nuance here.

Customer connectivity could reasonably be maintained through GIS/CIS/OMS integration in a real utility.

We do not need to introduce a full Customer Information System because the governing project does not require one.

So I recommend:

- GIS/network model contains the relationship between customer zones and network sections;

- OMS consumes that relationship to calculate outage impact.

This preserves the important concept without adding another unnecessary system.

### Engineering decision — ED-B07

Customer-zone-to-network connectivity shall be treated as maintained network information available to OMS; no standalone customer information system will be modelled.

That is a deliberate scope-control decision.

# B.14 End-to-End Information Flow

We can now construct our first credible conceptual flow.

## Normal network information

GIS

│

│ Network assets

│ Connectivity

│ Normal configuration

↓

ADMS

## Live operational state

Field / Protection

↓

SCADA

│

│ Breaker status

│ Switch status

│ Telemetry quality

│ Alarms/events

↓

ADMS

## Derived topology

GIS connectivity

+

SCADA states

↓

ADMS

↓

Operational topology

Section energisation

Restoration eligibility

## Outage/customer impact

ADMS

│

│ De-energised sections

↓

OMS

│

│ Customer-zone mapping

↓

Outage extent

Affected customers

This is the core information architecture for the project.

# B.15 Fault Workflow Across Systems

Now apply that architecture to our actual scenario.

### Step 1 — Fault occurs

Distribution network

↓

Protection operates

Protection engineering itself remains outside scope.

### Step 2 — Feeder breaker trips

SCADA receives the changed breaker indication.

Feeder A breaker:

CLOSED → OPEN

SCADA records the point/state change.

### Step 3 — Alarm/event generated

A relevant operational alarm/event becomes available to the operator and integrated systems.

This corresponds with the Project Definition requirement for correct alarm generation.

### Step 4 — ADMS receives state change

ADMS combines:

GIS connectivity

+

SCADA breaker state

and recalculates the operational topology.

### Step 5 — Sections become de-energised

ADMS determines which sections no longer have a valid source path.

For example:

A1 = DE-ENERGISED

A2 = DE-ENERGISED

A3 = DE-ENERGISED

### Step 6 — OMS determines outage impact

The affected sections are associated with customer zones.

A1 → CZ-A1

A2 → CZ-A2

A3 → CZ-A3

OMS calculates the affected customer total.

### Step 7 — Operator acknowledges alarm

SCADA/operational interface records acknowledgement.

The acknowledgement should generate an auditable operational event.

### Step 8 — Restoration assessed

ADMS evaluates:

- fault isolation;

- alternate supply path;

- telemetry validity;

- alternate-feeder capacity.

If permitted, restoration can be proposed.

### Step 9 — Switching changes topology

Following the approved simulated switching operation:

tie OPEN → CLOSED

ADMS recalculates topology.

Healthy isolated sections with a valid alternate-source path become energised.

### Step 10 — OMS updates outage

OMS recalculates current customer impact based on the new energisation state.

That means our outage information should change as network state changes rather than remain a static incident record.

# B.16 IQ-11 — Event Logging

The governing Vision says that every engineering action should be traceable through system logs.

There are really two kinds of traceability in this project, and we should distinguish them.

### Operational event history

What happened in the simulated operational environment?

Examples:

- breaker opened;

- alarm created;

- topology recalculated;

- outage identified;

- alarm acknowledged;

- restoration assessed;

- switching action performed;

- sections re-energised.

### Engineering validation history

What did the engineer do while validating the system?

Examples:

- test executed;

- expected result compared;

- validation failed;

- investigation started;

- configuration defect identified;

- correction applied;

- test repeated;

- test passed.

These are related, but they are not the same thing.

This distinction will give the final demonstrator much more credibility.

### Engineering decision — ED-B08

The project shall distinguish operational event records from engineering validation evidence, while maintaining traceability between them where relevant.

# B.17 Minimum Operational Event Record

For the demonstrator, I recommend each operational event contain:

- event ID;

- timestamp;

- source system;

- event type;

- affected asset/entity;

- description;

- previous value/state where applicable;

- new value/state where applicable;

- actor/origin where applicable.

Commercial SCADA event systems commonly associate records with timestamps and information describing the source and condition/activity, and use them to establish an audit trail.

We do not need to reproduce a vendor-specific event schema.

### Example

Event ID: EVT-0042

Timestamp: 10:32:14.218

Source: SCADA

Type: DEVICE_STATE_CHANGE

Asset: BRK-FDR-A

Previous: CLOSED

New: OPEN

Description: Feeder A breaker opened following protection operation

Then:

Event ID: EVT-0043

Timestamp: 10:32:14.315

Source: ADMS

Type: TOPOLOGY_UPDATED

Description: Feeder A downstream sections no longer connected to active source

Then:

Event ID: EVT-0044

Timestamp: 10:32:14.427

Source: OMS

Type: OUTAGE_IDENTIFIED

Description: 915 customers identified as affected

Notice what we’re demonstrating here:

one physical event produces consequences across several operational systems.

That is exactly the kind of systems thinking this project should showcase.

# B.18 Source-of-Truth Responsibilities

We can now establish a very useful preliminary responsibility table.

| Information | Conceptual owner/source |
| --- | --- |
| Asset identity | GIS |
| Configured network connectivity | GIS |
| Normal switch configuration | GIS/configuration |
| Current breaker state | SCADA |
| Current remotely monitored switch state | SCADA |
| Telemetry quality/timestamp | SCADA |
| Active operational topology | ADMS |
| Section energisation | ADMS |
| Restoration assessment | ADMS |
| Customer-zone/network relationship | Network/GIS information |
| Current outage extent | OMS |
| Affected customer count | OMS |
| Operational event history | Event/audit layer across systems |
| Engineering test outcome | Validation package |

This will be extremely valuable when we reach the System Architecture and data model.

# B.19 Important Defect Implication

Package B gives us a much stronger view of the possible intentional defect.

Consider this:

GIS configuration says:

Section A2 → Section A3

But the incorrect configured topology says:

Section A2 → Section B2

SCADA could still correctly report:

Feeder A breaker = OPEN

Yet ADMS would perform its topology calculation using incorrect connectivity.

The resulting downstream energisation or outage extent would then be wrong.

OMS could consequently produce an incorrect customer count.

So the engineering investigation could follow:

Correct SCADA state

↓

Unexpected ADMS topology

↓

Incorrect OMS outage result

↓

Engineer compares expected vs observed

↓

Checks breaker telemetry — correct

↓

Traces network model

↓

Finds GIS/configuration connectivity error

↓

Corrects configuration

↓

Reruns validation

↓

Correct topology + customer result

That is already looking like a very strong defect candidate because it traverses multiple systems and requires actual engineering reasoning rather than code debugging.

I’m still not locking it until Package C, but Package B has made it considerably more attractive.

# B.20 Investigation Conclusions

Package B gives us eight primary engineering decisions:

| ID | Decision |
| --- | --- |
| ED-B01 | Define OT systems by conceptual function rather than replicate a specific vendor architecture. |
| ED-B02 | SCADA represents monitored field states and operational events. |
| ED-B03 | SCADA provides observed states; network energisation is derived separately. |
| ED-B04 | GIS represents persistent asset/connectivity information; telemetry provides current device states. |
| ED-B05 | ADMS performs operational topology processing, energisation determination and simplified restoration assessment. |
| ED-B06 | OMS determines outage and customer impact from de-energised network sections. |
| ED-B07 | Customer-zone connectivity is represented through network/GIS information without adding a standalone CIS. |
| ED-B08 | Operational event history and engineering validation evidence shall be distinct but traceable. |

# B.21 Resulting Conceptual Architecture

Our current engineering understanding is therefore:

FICTIONAL DISTRIBUTION NETWORK

│

Field states

↓

SCADA

telemetry / alarms

│

↓

GIS ── connectivity/model ──→ ADMS

│

topology / energisation

│

┌──────────────┴─────────────┐

↓                            ↓

OMS                    Restoration logic

│

customer impact

↓

OPERATOR / VALIDATION

Across the workflow:

OPERATIONAL EVENT LOG

Separate but linked:

ENGINEERING VALIDATION RECORD

That is a sufficiently credible conceptual architecture for this work package without pretending that every utility deploys these products in exactly this way. Real implementations can consolidate several of these functions within a single ADMS platform, and NREL explicitly describes modern ADMS deployments as integrations of multiple utility systems and functions.

Package B is therefore complete enough for our purposes.

The next stage should be Package C — Fault & Restoration Workflow, and I expect it to be the most engineering-heavy investigation so far. That is where we need to establish the actual rules behind fault isolation, restoration eligibility, feeder-capacity checks and stale/uncertain telemetry. Those conclusions will determine the behaviour we later turn into formal requirements.

# Engineering Investigation C

## Fault, Restoration & Telemetry Quality

Investigation ID: EI-C
Status: Draft
Related Investigation Questions: IQ-02, IQ-09, IQ-10

# C.1 Purpose

The purpose of this investigation is to define the minimum credible operational rules governing the project’s feeder-fault and restoration scenario.

The governing Project Definition already fixes the high-level workflow:

Fault → protection trip → topology update → downstream de-energisation → alarm → outage identification → affected-customer calculation → operator acknowledgement → restoration proposal → capacity check → restoration permitted/rejected → event logging.

This investigation therefore does not redesign that sequence.

Instead, it answers three implementation-critical questions:

- IQ-02: What should happen operationally when the feeder fault occurs?

- IQ-09: What conditions must be satisfied before restoration is considered valid?

- IQ-10: How should stale or uncertain telemetry affect restoration and operational decisions?

The goal is to establish explicit engineering rules that can later become formal requirements and validation test cases.

# C.2 Investigation Boundary

This project is not performing protection-system studies, detailed fault calculations, full AC power-flow analysis or production FLISR optimisation.

Industry FLISR implementations can include fault location, isolation, reconfiguration analysis, alternate-path selection, feeder-capacity assessment and automated switching. NREL presents restoration logic in essentially that form: fault occurrence, isolation/reconfiguration analysis, checking switches and spare capacity, followed by load transfer and restoration where viable.

Our project needs only the subset necessary to demonstrate sound engineering reasoning:

detect changed state → derive outage → isolate fault → identify healthy restorable load → verify alternate supply and capacity → reject if information is unreliable → permit restoration if all conditions pass.

# C.3 IQ-02 — Behaviour Following the Fault

## Finding C1 — Protection operation is the initiating event, not the subject of validation

The governing scenario begins with a fault followed by the feeder breaker tripping.

As established in the Scope Brief, we assume that the protection system has correctly operated.

Therefore the validation package begins effectively at:

The feeder breaker has changed from CLOSED to OPEN following protection operation.

We are not required to calculate fault current or prove that the breaker should have tripped.

### Engineering decision — ED-C01

The feeder fault shall be represented as an initiating scenario condition that results in the relevant feeder breaker opening; detailed protection operation shall remain outside scope.

# C.4 Immediate Operational Consequence

After the breaker opens, the operational environment should not simply set a pre-programmed outage.

Package A established that section energisation is derived from topology, and Package B established that SCADA reports the breaker state while ADMS derives the network consequences.

Therefore:

Fault occurs

↓

Protection trips breaker

↓

SCADA observes OPEN

↓

ADMS receives new device state

↓

Topology recalculated

↓

Sections without active source path become de-energised

↓

OMS determines customer impact

This is important because it means the fault does not directly create the outage in software logic.

The changed network state creates the outage.

### Engineering decision — ED-C02

Loss of supply shall be determined from the network topology resulting from the breaker state change rather than from a hard-coded association between the fault event and affected sections.

# C.5 Faulted Section Versus De-Energised Sections

These concepts also need to remain separate.

Suppose a fault occurs on Section A2.

After the feeder breaker trips, it is entirely possible that:

- A1 is de-energised;

- A2 is de-energised and faulted;

- A3 is de-energised;

even though only A2 actually contains the fault.

So:

faulted describes the location of the abnormal electrical condition.

de-energised describes the operational supply state.

A healthy section can therefore be de-energised as a consequence of protection operation.

This distinction is fundamental to restoration, because healthy de-energised sections may be restorable while the faulted section must remain isolated.

### Engineering decision — ED-C03

The network model shall distinguish fault status from energisation status. A section may be healthy but de-energised, while the identified faulted section shall remain unavailable for restoration.

# C.6 Fault Isolation

Investigation A identified that alternate supply must not re-energise the fault.

Industry distribution-restoration practice relies on sectionalising devices and normally-open ties so that the faulted portion can remain isolated while healthy portions are restored from another feeder. EPRI describes radial feeders divided into load sections using sectionalising switches, with normally-open ties used to transfer disconnected load to supporting feeders following system failures.

This means our final network must contain enough switching capability to create three states:

SOURCE A → healthy section → FAULTED SECTION → healthy downstream section

After protection trip:

all affected portion de-energised

After isolation:

healthy upstream portion | OPEN | FAULTED SECTION | OPEN | healthy downstream portion

Then, if appropriate:

healthy downstream portion ← TIE ← SOURCE B

The precise network arrangement remains a Network Model decision, but the operational rule can now be locked.

### Engineering decision — ED-C04

The faulted section must be electrically isolated from any proposed restoration path before alternate supply is permitted.

This formalises RI-A01 from Investigation A.

# C.7 Restoration Is Not Equivalent to Closing the Tie

This is another important distinction.

The tie switch is only a means of establishing an alternate path.

Closing it is not automatically valid.

A credible restoration decision has several prerequisites.

NREL’s simplified FLISR logic explicitly checks whether controllable switching is available and whether the tie path has spare capacity before load transfer/restoration proceeds.

For our project, I recommend that restoration eligibility be expressed as a set of explicit conditions.

# C.8 IQ-09 — Restoration Eligibility

A restoration proposal should be considered valid only where all of the following are true.

## Condition 1 — Fault is isolated

No energised path may pass through the faulted section.

fault_isolated = TRUE

If false:

RESTORATION REJECTED

Reason: faulted section not isolated

## Condition 2 — Alternate source is available

The alternate feeder must itself be energised and capable of supplying through the proposed tie path.

Conceptually:

Feeder B source available

+

Feeder B breaker closed

+

valid path to tie

must all be true.

### Engineering decision — ED-C05

Restoration shall require a valid active path from an available alternate source to the healthy de-energised section.

# C.9 Condition 3 — Proposed Path Must Not Create an Invalid Network Configuration

Our simplified system normally operates radially.

We therefore should not casually close a tie while both feeder-side paths remain simultaneously energised if doing so creates an unintended parallel/looped supply configuration.

EPRI’s DMS guidance specifically treats locating loops and alternate feeder supply paths as important topology functions.

For this project, we do not need to analyse parallel power flows.

The simpler and safer project rule is:

The restoration configuration must preserve the intended radial operating arrangement.

### Engineering decision — ED-C06

A restoration action shall be rejected if the proposed switching state creates an unintended energised loop within the simplified network model.

That is enough for our educational scope.

# C.10 Condition 4 — Alternate Feeder Capacity

We need to evaluate:

- existing loading on the alternate feeder;

- load proposed for transfer;

- resulting alternate-feeder loading;

- defined feeder capacity.

We can formalise that now.

Let:

L_B = \text{existing load on alternate feeder}

L_T = \text{load to be transferred}

C_B = \text{alternate feeder capacity}

Then:

L_{result} = L_B + L_T

and:

Loading\% = \frac{L_{result}}{C_B}\times100

Restoration is permitted on capacity grounds if:

L_{result} \leq C_B

or equivalently:

Loading\% \leq 100\%

For example, if later we choose:

Feeder B existing load = 3.8 MW

Transfer load = 1.4 MW

Feeder B capacity = 6.0 MW

then:

3.8+1.4=5.2\text{ MW}

5.2/6.0\times100=86.7\%

The capacity test passes.

By contrast:

Existing load = 5.0 MW

Transfer = 1.4 MW

Capacity = 6.0 MW

gives:

6.4/6.0\times100=106.7\%

and restoration must be rejected.

Distribution-restoration literature likewise treats spare capacity in the alternate feeder as a fundamental limitation on load transfer.

### Engineering decision — ED-C07

Restoration shall be rejected where existing alternate-feeder load plus proposed transferred load exceeds the defined alternate-feeder capacity.

# C.11 Do We Need a Safety Margin?

A real utility might not permit operation right up to a theoretical 100% rating. Operating margins, emergency ratings, voltage constraints and other limits may matter.

However, the Project Definition explicitly calls for a simplified calculation based on feeder loading, transferable load, alternate feeder capacity and resulting loading percentage.

I therefore recommend not inventing a 90% or 80% engineering margin unless later evidence or project requirements specifically justify one.

For this demonstrator:

feeder capacity = the defined allowable restoration limit.

That keeps the calculation transparent and traceable to the governing scope.

### Engineering decision — ED-C08

The simplified restoration capacity criterion shall use the defined feeder capacity directly; no additional operating margin will be introduced unless subsequently justified and documented.

# C.12 What Is the Transferable Load?

This should also be derived rather than arbitrary.

The transferred load should equal the aggregated simplified load of the healthy de-energised sections that would become supplied through the alternate path.

For example:

A1 load = 0.8 MW

A2 load = 1.0 MW  ← faulted

A3 load = 1.4 MW

If A2 is isolated and only A3 is restorable through the tie:

L_T = 1.4\text{ MW}

not:

0.8 + 1.0 + 1.4

The capacity calculation should therefore depend directly on topology and isolation state.

### Engineering decision — ED-C09

Transferred load shall be calculated from the aggregate simplified load of healthy de-energised sections included in the proposed alternate-supply topology.

That links:

topology → restoration extent → transferable load → capacity decision.

# C.13 Condition 5 — Telemetry Must Be Trustworthy

Now we reach IQ-10.

The Project Vision explicitly states:

uncertain or stale telemetry prevents unsafe actions.

This is not just a software validation issue. It is an OT engineering issue.

A device state usually has more than just a value.

Conceptually:

Value: CLOSED

Timestamp: 10:31:42

Quality: GOOD

Schneider Electric’s SCADA documentation describes point values together with current quality and timestamp, and advises using quality/status information to assess reliability. It also describes data with invalid timestamps as suspect rather than simply treating the value as unquestionably current.

Other SCADA documentation explicitly recognises data-quality states such as Good, Uncertain, Bad, Last Known Value, Communication Failure, and related statuses.

Therefore our telemetry model should not be:

switch_state = CLOSED

It should conceptually be:

value = CLOSED

quality = GOOD

timestamp = 10:31:42

### Engineering decision — ED-C10

Operational telemetry used for restoration decisions shall include value, quality and timestamp information rather than state value alone.

# C.14 Simplified Telemetry Quality Model

We do not need dozens of OPC quality codes.

That would be unnecessary implementation detail.

I recommend only three project-level quality states:

### GOOD

The value is considered reliable and sufficiently current for operational use.

### UNCERTAIN

A value exists, but there is reason not to rely on it for safety-relevant restoration decisions.

### BAD

The value is unavailable or unreliable and shall not be used as evidence of current equipment state.

This mirrors the broad Good/Uncertain/Bad concept used in industrial telemetry while remaining easy to understand and test.

### Engineering decision — ED-C11

The demonstrator shall use a simplified telemetry-quality model of GOOD, UNCERTAIN and BAD.

# C.15 Staleness Is Different From Bad Quality

This distinction is worth preserving.

Consider:

Tie switch state = OPEN

Quality = GOOD

That may have been good information when observed.

But if the last update was 40 minutes ago, we may no longer be comfortable relying on it during restoration.

SCADA systems commonly use timestamps and configurable maximum-age concepts to determine whether cached information is sufficiently recent. Schneider Electric’s OPC UA documentation, for example, explicitly checks timestamp age against a configured Maximum Age.

Therefore:

Quality tells us whether the reported value was considered reliable.

Freshness tells us whether that information is recent enough for the current operational decision.

### Engineering decision — ED-C12

Telemetry validity for restoration shall consider both reported quality and data age.

# C.16 What Should Count as “Stale”?

We now need to be careful.

There is no universal industry value such as:

“SCADA data is stale after exactly 30 seconds.”

Different points, systems and operational procedures can have different update expectations.

So I do not recommend pretending that our eventual threshold is an industry standard.

Instead, we should create a documented fictional project criterion.

For example, later we might define:

Age = CurrentTime - TelemetryTimestamp

and:

if age <= STALE_THRESHOLD

telemetry_fresh = TRUE

else

telemetry_fresh = FALSE

The actual threshold—perhaps 30 seconds, 60 seconds or another value—can be chosen during the Engineering Design Brief/Requirements stage and explicitly identified as a project assumption.

### Engineering decision — ED-C13

Telemetry staleness shall be determined against a documented project-specific maximum-age threshold; the threshold shall not be presented as a universal utility standard.

This is exactly the kind of distinction that makes the engineering documentation credible.

# C.17 Which Telemetry Must Be Valid?

We should not require every telemetry point in the fictional utility to be fresh.

Only points necessary to establish the proposed switching condition should block restoration.

For our simplified scenario, that will likely include:

- alternate feeder breaker state;

- relevant sectionalising switch states;

- tie-switch state;

- possibly the original feeder breaker state.

If any required point has:

quality != GOOD

or:

age > allowed maximum

then the system should not declare restoration safe.

### Engineering decision — ED-C14

Restoration eligibility shall require valid telemetry for every monitored switching device whose state is necessary to establish the proposed restoration topology.

# C.18 What Should Happen When Telemetry Is Uncertain?

This is an important UX and engineering decision.

The system should not infer:

“Probably closed.”

It should also not simply crash the workflow.

Instead:

Restoration assessment:

BLOCKED

Reason:

Telemetry for SW-A2 is stale.

Last known state:

OPEN

Last update:

10:31:42

Current age:

128 s

This preserves situational awareness while refusing to treat the last-known state as adequate engineering evidence.

### Engineering decision — ED-C15

Where required telemetry is stale, UNCERTAIN or BAD, the restoration assessment shall be blocked and shall identify the telemetry condition responsible.

That directly implements the governing Vision’s requirement that uncertainty prevent unsafe actions.

# C.19 Proposal Versus Execution

There is another useful boundary we should establish.

The governing workflow says:

System proposes restoration → capacity check performed → restoration permitted or rejected.

So the system can conceptually identify a candidate restoration path before all permissive checks have succeeded.

For example:

Candidate:

Restore A3 via Feeder B / tie TS-01

Then:

Fault isolation: PASS

Alternate source: PASS

Topology validity: PASS

Telemetry: PASS

Capacity: PASS

Result:

RESTORATION PERMITTED

Or:

Telemetry: FAIL

Result:

RESTORATION BLOCKED

This gives us an excellent opportunity in the demonstrator to show engineering reasoning rather than simply a green/red button.

### Engineering decision — ED-C16

The demonstrator shall distinguish a candidate restoration proposal from the subsequent engineering assessment that determines whether that proposal is permitted or rejected.

# C.20 Restoration Decision Logic

We can now define a preliminary project rule.

Let:

- I = fault isolated;

- S = alternate source available;

- R = resulting topology valid/radial;

- T = required telemetry valid;

- C = capacity check passed.

Then:

RestorationPermitted = I \land S \land R \land T \land C

In plain language:

IF fault isolated

AND alternate supply available

AND restoration topology valid

AND required telemetry trustworthy

AND feeder capacity sufficient

THEN

restoration permitted

ELSE

restoration rejected/blocked

This becomes a very strong piece of engineering logic because every condition has an identifiable basis and can be independently tested.

### Engineering decision — ED-C17

Restoration shall be permitted only when all defined restoration permissive conditions are satisfied.

# C.21 Failure Reason Must Be Visible

If restoration is not permitted, the system should not output only:

RESTORATION FAILED

That provides very little engineering information.

It should identify the failed criterion.

For example:

RESTORATION REJECTED

Capacity check:

Existing Feeder B load: 5.0 MW

Transfer load: 1.4 MW

Resulting load: 6.4 MW

Feeder B capacity: 6.0 MW

Loading: 106.7%

Reason:

Alternate feeder capacity exceeded.

or:

RESTORATION BLOCKED

Reason:

Tie-switch telemetry stale.

Last update age: 92 seconds

Maximum permitted age: 60 seconds.

### Engineering decision — ED-C18

A rejected or blocked restoration assessment shall identify the failed permissive condition and provide the relevant supporting information.

This will make validation much easier because expected and observed behaviour can be compared directly.

# C.22 Capacity Failure Versus Telemetry Block

I recommend distinguishing two outcomes semantically.

### REJECTED

The information is sufficient to evaluate the proposal and the engineering criterion fails.

Example:

feeder overload.

### BLOCKED

The system cannot safely determine whether restoration is permissible because required information is inadequate.

Example:

stale tie-switch telemetry.

This is more precise than treating everything as “failed.”

### Engineering decision — ED-C19

Restoration assessment shall distinguish between REJECTED outcomes caused by failed engineering criteria and BLOCKED outcomes caused by insufficient or unreliable operational information.

That distinction could become one of the more thoughtful parts of the project.

# C.23 Resulting Fault and Restoration Workflow

We can now expand the governing workflow into an engineering-level sequence.

1. Fault occurs on distribution section

↓

2. Protection trips feeder breaker

↓

3. SCADA receives OPEN breaker status

↓

4. Operational alarm/event generated

↓

5. ADMS recalculates topology

↓

6. Sections without valid source path

become DE-ENERGISED

↓

7. OMS calculates affected customers

↓

8. Operator acknowledges alarm

↓

9. Fault location is identified for scenario

↓

10. Faulted section isolated

↓

11. Healthy de-energised sections identified

↓

12. Candidate alternate-supply path determined

↓

13. Restoration permissives evaluated:

• fault isolated?

• alternate source available?

• topology valid?

• telemetry valid?

• feeder capacity sufficient?

↓

┌────┴────┐

│         │

YES        NO

│         │

↓         ↓

PERMITTED   REJECTED/

BLOCKED

│

↓

14. Simulated switching performed

↓

15. Network topology recalculated

↓

16. Healthy sections re-energised

↓

17. OMS recalculates affected customers

↓

18. Operational actions logged

This is not replacing the Project Definition’s workflow.

It is the engineering decomposition of the existing workflow into behaviours we can specify and validate.

# C.24 Important Scope Control — Fault Location

You might notice Step 9 says:

fault location is identified for scenario.

We need that qualifier.

Actual automated fault-location algorithms can involve protection measurements, fault indicators, impedance estimates and other data.

That is beyond scope.

The project only needs a known faulted section so that we can validate isolation and restoration behaviour.

Therefore:

### Engineering decision — ED-C20

The faulted section shall be predefined or otherwise provided as part of the validation scenario; development of a fault-location algorithm is outside scope.

That prevents another large area of feature creep.

# C.25 Important Scope Control — Switching Authority

Similarly, the application should not imply that a junior engineer or an autonomous algorithm can physically energise a utility network.

Our demonstrator is simulation only.

So I recommend the following representation:

System proposes restoration

↓

Assessment passes

↓

Restoration PERMITTED

↓

Operator initiates simulated switching

We can later decide exactly how this appears in the UI, but conceptually we should preserve human/operator authority.

### Engineering decision — ED-C21

A successful restoration assessment shall indicate that simulated restoration is permitted; it shall not be represented as autonomous authority to operate real electrical equipment.

This aligns well with the graduate-level, validation-focused scope.

# C.26 Investigation C Conclusions

Package C produces the following engineering decisions:

| ID | Decision |
| --- | --- |
| ED-C01 | Protection trip is an initiating scenario condition; detailed protection analysis is outside scope. |
| ED-C02 | Outage state is derived from topology after breaker operation. |
| ED-C03 | Fault status and energisation status are separate concepts. |
| ED-C04 | Faulted section must be isolated before alternate restoration. |
| ED-C05 | Restoration requires a valid alternate-source path. |
| ED-C06 | Restoration must preserve intended radial topology and avoid unintended energised loops. |
| ED-C07 | Alternate feeder must have sufficient capacity after load transfer. |
| ED-C08 | Defined feeder capacity is used directly without an invented extra operating margin. |
| ED-C09 | Transferable load is derived from healthy sections included in the restoration path. |
| ED-C10 | Operational telemetry includes value, quality and timestamp. |
| ED-C11 | Simplified telemetry quality states are GOOD, UNCERTAIN and BAD. |
| ED-C12 | Telemetry validity considers both quality and freshness. |
| ED-C13 | Staleness uses a documented project-specific threshold. |
| ED-C14 | All telemetry required to establish restoration topology must be valid. |
| ED-C15 | Invalid/stale required telemetry blocks restoration. |
| ED-C16 | Candidate restoration proposal and permissive assessment are distinct. |
| ED-C17 | All restoration permissive conditions must pass. |
| ED-C18 | Rejected/blocked decisions identify the failed criterion and supporting evidence. |
| ED-C19 | REJECTED and BLOCKED represent different failure classes. |
| ED-C20 | Fault location is provided by the scenario; no fault-location algorithm is developed. |
| ED-C21 | Restoration permission supports simulated operator action rather than autonomous real-world control. |

# C.27 Requirements Emerging From Investigation C

We’re not writing the Requirements Specification yet, but several clear requirement candidates have now emerged:

Topology shall be recalculated following relevant switching-state changes.

Faulted sections shall remain isolated during restoration.

Restoration shall require an available alternate source.

Restoration shall not create an unintended energised network loop.

Restoration shall be rejected where alternate feeder capacity would be exceeded.

Operational telemetry used in restoration assessment shall include quality and timestamp information.

Restoration shall be blocked where required telemetry is stale or unreliable.

Restoration assessment shall identify the reason for rejection or blocking.

Successful restoration shall trigger recalculation of section energisation and affected-customer count.

These are exactly the kinds of statements we will formalise later into requirements IDs such as REQ-TOP-001, REQ-RST-001, etc.

# C.28 Implication for Intentional Defect Selection

Packages A–C now give us enough context to choose the seeded defect intelligently.

The four governing candidates were:

- incorrect switch normal state;

- incorrect topology connection;

- stale telemetry;

- inconsistent configuration data.

At this point, I would rank them:

1. Incorrect topology/configuration connection — strongest candidate.
It can cause correct SCADA telemetry to produce incorrect downstream energisation and customer impact. Investigation therefore crosses GIS/configuration → ADMS topology → OMS outage calculation. That demonstrates systems reasoning and root-cause analysis very well.

2. Stale telemetry — also strong.
It demonstrates a genuinely OT-specific concept: the value may look correct while being untrustworthy because of quality/age. It is excellent for showing operational safety reasoning, but the root-cause investigation itself may be slightly shorter.

3. Incorrect switch normal state.
Credible and easy to explain, but potentially too obvious once the engineer inspects the network configuration.

4. Generic inconsistent configuration data.
Too vague unless we define exactly what configuration is inconsistent; effectively it would probably become one of the first three anyway.

So Package C has confirmed that incorrect topology connectivity is probably the best seeded engineering defect for this particular work package, while stale telemetry should remain a normal negative validation scenario rather than necessarily being the intentional defect.

I would still make that selection formally in Package D, because Package D should record not merely what defect we chose, but why that defect gives the strongest validation and investigation exercise relative to the alternatives.

# Engineering Investigation D

## Intentional Defect Selection

Investigation ID: EI-D
Status: Draft
Related Investigation Question: IQ-12

# D.1 Purpose

The purpose of this investigation is to select the single intentional engineering defect that will be seeded into the project to produce a realistic failed validation scenario.

The governing Project Definition requires one intentional defect and identifies several candidate categories:

- incorrect switch normal state;

- incorrect topology connection;

- stale telemetry;

- inconsistent configuration data.

The defect must support the complete engineering workflow expected of the graduate engineer:

failed validation → investigation → root cause identification → correction → repeat validation → successful verification.

The defect should therefore be selected on engineering value, not on which software bug is easiest to implement.

# D.2 Selection Criteria

The chosen defect should satisfy the following criteria.

## D.2.1 Relevance to the Core Scenario

The defect should materially affect one or more of the behaviours already defined as validation objectives:

- topology update;

- section energisation;

- outage identification;

- affected-customer calculation;

- restoration validity;

- operational traceability.

A defect that affects an irrelevant UI field or non-essential software component would not provide meaningful engineering evidence.

## D.2.2 Plausibility as an OT Configuration/Data Issue

The defect should resemble the kind of issue that could realistically arise during migration, configuration, integration or commissioning of an operational environment.

The governing scenario explicitly frames the graduate role around tracing configuration and data issues rather than debugging arbitrary application code.

## D.2.3 Investigability

The initial symptom should not immediately reveal the root cause.

The engineer should need to compare:

- expected behaviour;

- observed behaviour;

- telemetry;

- network configuration;

- derived system state;

- and relevant data relationships.

This allows the investigation to demonstrate structured reasoning.

## D.2.4 Cross-System Impact

A stronger defect will propagate across conceptual system boundaries.

For example:

incorrect configuration

↓

incorrect topology processing

↓

incorrect energisation state

↓

incorrect outage/customer result

That supports the systems-thinking objective of the project better than an isolated software error.

## D.2.5 Correctability

The defect should be correctable through a clearly identifiable configuration or data change rather than requiring redesign of the whole demonstrator.

This is important because the exercise is intended to represent a manageable graduate validation package.

## D.2.6 Verifiability

After correction, the exact failed test should be rerun and should produce a clearly different, correct outcome.

The project should be able to show evidence of both:

before correction — FAIL

and:

after correction — PASS

## D.2.7 Scope Discipline

The defect should remain understandable without introducing major additional functionality.

A defect that requires advanced protection engineering, real-time communication simulation or power-flow modelling would undermine the project’s deliberately simplified scope.

# D.3 Candidate 1 — Incorrect Switch Normal State

A switch configuration could be stored with the wrong normal state.

For example:

Expected:

TS-01 normal state = OPEN

Configured:

TS-01 normal state = CLOSED

This could produce an unexpected topology during model initialisation or restoration assessment.

### Strengths

It is credible as a configuration issue and directly relates to switching and topology.

It would be relatively easy to demonstrate and correct.

### Weaknesses

The root cause may be too obvious.

Once the engineer notices that the tie switch appears closed when the network is expected to operate radially, the investigation may collapse into:

“The switch state is wrong → change it.”

That provides limited opportunity to demonstrate deeper cross-system investigation.

There is also a risk of confusing normal/configured state with current SCADA state, which we deliberately separated in Investigation B.

### Assessment

Suitable, but not preferred.

# D.4 Candidate 2 — Stale Telemetry

A relevant switch or breaker state could have a valid-looking last-known value but an old timestamp.

Example:

Tie switch TS-01

Value: OPEN

Quality: GOOD

Timestamp: 10:20:00

Assessment time: 10:25:00

Maximum permitted age: 60 s

The restoration assessment would therefore be blocked despite the displayed state appearing plausible.

### Strengths

This is strongly OT-specific.

It demonstrates that a telemetry value cannot be treated independently of its quality and timestamp.

It also supports the Project Vision’s explicit requirement that stale or uncertain telemetry prevent unsafe action.

### Weaknesses

Once the validation output states:

“Restoration blocked: stale telemetry,”

the root cause may already be largely exposed.

The investigation would likely focus on a single telemetry record rather than requiring the engineer to trace behaviour across GIS/configuration, ADMS topology and OMS outage processing.

### Assessment

Excellent negative validation case, but not the strongest intentional defect.

I recommend we definitely test stale telemetry later, just not use it as the central defect investigation.

# D.5 Candidate 3 — Incorrect Topology Connection

A network connectivity relationship is configured incorrectly.

For example, the intended model might contain:

Section A1

↓

Section A2

↓

Section A3

but the configured model could incorrectly contain:

Section A1

↓

Section A2

Section A3 ← incorrect upstream relationship

or another plausible misconnection depending on the final Network Model.

Crucially, live telemetry could still be completely correct.

For example:

Feeder A breaker = OPEN

Quality = GOOD

Timestamp = current

SCADA therefore behaves correctly.

However, ADMS consumes incorrect configured connectivity and derives the wrong operational topology.

That can then cause OMS to produce incorrect outage extent or affected-customer information.

# D.6 Expected Failure Chain

The defect could propagate as follows:

Incorrect network connection

↓

ADMS topology trace uses incorrect connectivity

↓

Incorrect section energisation state

↓

Incorrect de-energised section set

↓

OMS receives wrong outage extent

↓

Incorrect affected-customer calculation

↓

Validation test FAILS

This gives us a strong multi-stage symptom while keeping the actual defect small.

# D.7 Investigation Path

A believable graduate investigation could proceed like this:

### Step 1 — Observe Failed Validation

The expected affected-customer count does not match the observed count.

Example only:

Expected affected customers: 720

Observed affected customers: 495

Result: FAIL

The exact values will be established later in the Network Model.

### Step 2 — Confirm Initiating Telemetry

Check the feeder breaker state.

Feeder A breaker:

OPEN

Quality: GOOD

Timestamp: valid

Conclusion:

SCADA correctly represents the feeder trip.

This eliminates an obvious telemetry failure.

### Step 3 — Compare Expected and Observed Energisation

The engineer examines section state.

For example:

Expected:

A1 = DE-ENERGISED

A2 = DE-ENERGISED

A3 = DE-ENERGISED

Observed:

A1 = DE-ENERGISED

A2 = DE-ENERGISED

A3 = ENERGISED

This narrows the problem from customer calculation to network topology.

### Step 4 — Verify Customer Mapping

The customer-zone association for A3 is checked.

If:

A3 → CZ-A3 → 225 customers

is correct, then OMS is not necessarily the root cause.

It is simply using the state information supplied to it.

That distinction is valuable.

### Step 5 — Inspect Network Connectivity

The engineer traces A3 through the configured network model and discovers that its upstream connection is incorrect.

The configuration may incorrectly give A3 a path to an energised source.

### Step 6 — Identify Root Cause

The defect is recorded as a network-model configuration error.

For example:

Section A3 was incorrectly configured with an upstream connection to Section B3, causing ADMS topology processing to treat A3 as energised following the Feeder A breaker trip.

Again, the exact connection should only be finalised once we design the actual network.

### Step 7 — Correct Configuration

The incorrect relationship is replaced with the intended connection.

No application algorithm is changed.

This is important.

The defect is a configuration defect, not a failure in the topology engine itself.

### Step 8 — Repeat Validation

The same test is executed with the corrected configuration.

Expected affected customers: 720

Observed affected customers: 720

Result: PASS

### Step 9 — Verify Downstream Effects

The engineer verifies not just the final customer number, but the whole corrected chain:

Breaker state correct

↓

Topology correct

↓

Section energisation correct

↓

Outage extent correct

↓

Customer calculation correct

That produces far stronger validation evidence than simply showing that a UI value changed.

# D.8 Candidate 4 — Generic Inconsistent Configuration Data

The Project Definition also allows “inconsistent configuration data.”

This could represent many things:

- feeder association mismatch;

- inconsistent asset identifier;

- customer-zone association mismatch;

- device configuration disagreement;

- duplicated connectivity data;

- capacity data mismatch.

### Strengths

Potentially realistic and flexible.

### Weaknesses

As currently expressed, it is too broad.

We would eventually have to turn it into a specific configuration defect anyway.

An incorrect topology connection is therefore essentially a specific, technically meaningful form of inconsistent configuration data.

### Assessment

Too generic as the final defect definition.

# D.9 Candidate Comparison

| Criterion | Switch Normal State | Stale Telemetry | Incorrect Topology Connection | Generic Config |
| --- | --- | --- | --- | --- |
| Core scenario relevance | High | High | High | Variable |
| OT plausibility | High | High | High | High |
| Investigation depth | Medium | Medium | High | Variable |
| Cross-system reasoning | Medium | Medium | High | Variable |
| Clear correction | High | High | High | Variable |
| Repeat-test value | High | High | High | High |
| Scope compatibility | High | High | High | Variable |
| Root cause not immediately obvious | Medium | Low–Medium | High | Variable |

The incorrect topology connection provides the strongest overall match.

# D.10 Selected Intentional Defect

### Engineering decision — ED-D01

The project’s intentional engineering defect shall be an incorrect network topology connection within the configured distribution network model.

The precise misconfigured relationship will be defined during development of the Network Model, once the final section and switching arrangement has been established.

The defect shall be designed so that:

- SCADA telemetry remains correct;

- the topology engine operates according to its configured data;

- the incorrect configuration causes at least one incorrect section energisation result;

- the incorrect energisation propagates into an incorrect outage/customer-impact result;

- the failed validation can be traced back to the topology configuration;

- correcting the configuration resolves the issue without modifying the topology-processing algorithm.

# D.11 Why This Is the Right Defect

The value of this defect is that none of the conceptual systems necessarily has to be “broken.”

SCADA may correctly report the field condition.

ADMS may correctly execute its topology logic.

OMS may correctly calculate customer impact from the energisation information it receives.

Yet the overall result is wrong because the underlying configuration is wrong.

That illustrates an important systems-engineering lesson:

Correct software processing does not guarantee a correct operational result if the configuration and source data are incorrect.

That is highly aligned with the graduate responsibilities established by the Project Definition, which include tracing configuration and data issues, determining root causes, recommending corrections and verifying the corrected system.

# D.12 Defect Type

The defect should formally be classified as:

Category: Configuration / network-model data defect
Domain: Distribution network topology
Detected through: Engineering validation
Primary symptom: Incorrect derived network state
Downstream impact: Incorrect outage/customer-impact result
Correction type: Configuration correction
Verification: Repeat execution of failed validation test

This classification can later appear in the Defect Investigation Report.

# D.13 What We Should Not Do

We should avoid implementing the defect as hidden application code such as:

if defect_mode:

customer_count = wrong_number

That would make the final demonstration visually work but destroy the engineering value.

The wrong outcome must arise naturally because the topology algorithm processes incorrect configuration.

In other words:

BAD:

Defect flag → deliberately display wrong answer

versus:

GOOD:

Incorrect configured connection

↓

normal topology algorithm

↓

wrong derived energisation

↓

normal outage calculation

↓

wrong customer result

The second creates an actual investigable engineering system.

### Engineering decision — ED-D02

The intentional failure shall be produced through erroneous configuration data processed by normal system logic, not through special-case code that directly forces a failed result.

# D.14 Defect Introduction and Correction

There is also a configuration-management issue to consider.

The final project should preserve evidence of both:

- the defective baseline configuration, and

- the corrected configuration.

Otherwise, once we fix the defect, the evidence of the original problem disappears.

I recommend eventually maintaining versions such as:

Network Model Configuration

Version 1.0 — Validation baseline

Contains intentional topology defect

Version 1.1 — Corrected configuration

Topology defect corrected following investigation

This could be represented through versioned data files, database records or controlled seed configurations; we don’t need to choose the technical implementation yet.

### Engineering decision — ED-D03

The project shall preserve identifiable defective and corrected configuration states to support repeatable validation and configuration traceability.

That will also support the project’s configuration-management learning objective.

# D.15 Stale Telemetry Retained as a Validation Case

Rejecting stale telemetry as the intentional defect does not mean we discard it.

It is explicitly part of the Project Vision and should still be validated.

Therefore:

### Engineering decision — ED-D04

Stale or unreliable telemetry shall be retained as a defined negative validation condition demonstrating that restoration is blocked when required operational information cannot be trusted.

This gives us two different and complementary demonstrations:

### Intentional engineering defect

configuration looks plausible

↓

system produces incorrect result

↓

engineer investigates

↓

root cause found and corrected

### Negative functional validation

telemetry intentionally stale

↓

system detects invalid information

↓

restoration correctly BLOCKED

↓

test PASS

That distinction is excellent.

The first demonstrates defect investigation.

The second demonstrates correct safety-oriented system behaviour.

# D.16 Investigation D Conclusions

| ID | Decision |
| --- | --- |
| ED-D01 | Use an incorrect network topology connection as the single seeded engineering defect. |
| ED-D02 | The defect shall exist in configuration data and be processed by normal topology logic; no artificial “wrong result” code will be used. |
| ED-D03 | Preserve defective and corrected configuration states for traceability and repeat validation. |
| ED-D04 | Retain stale telemetry as a separate negative validation scenario rather than the seeded defect. |

# D.17 Final Step 2 Decision Set

With Package D complete, our Step 2 investigation has now established the engineering foundations for the project.

At a high level:

Network

- normally radial;

- two feeders with a normally-open tie;

- sections and customer zones;

- dynamic topology-derived energisation.

Operational systems

- GIS/configuration supplies network connectivity;

- SCADA supplies live monitored state;

- ADMS derives topology and assesses restoration;

- OMS derives outage/customer impact.

Restoration

- fault must be isolated;

- alternate source must be available;

- topology must remain valid;

- telemetry must be trustworthy;

- alternate feeder must have capacity.

Validation

- operational events and engineering evidence are distinct but traceable;

- stale telemetry is a negative functional test;

- topology configuration error is the seeded defect.

Defect

- correct telemetry;

- incorrect topology configuration;

- wrong derived energisation/customer impact;

- engineering investigation;

- configuration correction;

- exact test repeated;

- PASS after remediation.

This is now a coherent engineering basis for moving forward.

## Step 2 — Engineering Investigation & Research: Complete

I would consider Step 2 complete at this point. We have enough research-backed decisions to stop investigating and start designing. Continuing general research now would likely have diminishing returns and risks turning the project into a theory exercise.

The next lifecycle stage should be Step 3 — Engineering Design Brief.

That document should convert everything we’ve learned in A–D into a single proposed engineering solution: what we’re going to build, how the conceptual systems fit together, what the network must support, how restoration will work, where the seeded defect sits, and why those design choices are appropriate.

Crucially, the Design Brief should still stay one level above the detailed Requirements Specification and Network Model. We know enough now to say how the solution should work, but we don’t yet need to choose exact section IDs, customer counts, MW values or database tables.